SECTION 08620

PLASTIC GLAZED UNIT SKYLIGHTS

Deck Mount with Multiwall Lumira panels

PART 1-GENERAL

  - SUMMARY

- This section includes the following:

  - Deck-mounted unit skylights with multiwall panel filled with Lumira™ aerogel.

    - CLD2 model standard single dome over multiwall Lumira™ aerogel filled panel.

    - CMD2 model pyramid single dome over multiwall Lumira™ aerogel filled panel.

- Related Sections:  The following sections contain requirements that relate to this section:

  - Division Six Section “Rough Carpentry” for wood curbs and nailers.

  - Division Seven Section “Roof Accessories” for curbs, roof hatches, and smoke vents.

  - Division Seven Section “Flashing and Sheet Metal” for metal flashing for skylights.

C.  Refer to roofing system sections for roofing accessories to be built into the roofing system to          	accommodate work of this section.

  - PERFORMANCE REQUIREMENTS

- General:  Provide unit skylights capable of withstanding loads as prescribed by the prevailing code for the project location.

- Unit skylights must be tested in accordance with AAMA\WDMA\CSA\101\I.S.2\A440 as required by Section 2405.5 of the International Building Code.

1.3 SUBMITTALS

- Product Data Sheet:  For each type of skylight specified, include details of construction and installation, relative to applicable roofing materials.

- Samples for Selection:  Manufacturer’s color charts showing a full range of colors available for each type of skylight glazing and aluminum finish.

  - QUALITY ASSURANCE

- Fire-Test-Response Characteristics:

  - Provide thermoformed domes fabricated from sheets identical to those tested for the following fire-test-response characteristics, per ASTM test method indicated below, by UL or other testing and inspecting agencies acceptable to authorities having jurisdiction.  Identify plastic sheets with appropriate markings of applicable testing and inspecting organization.

    - Self-Ignition Temperature:  651 deg F (343 deg C) or greater when tested per ASTM D 1929 on plastic sheets in the thickness intended for use.

    - Smoke density of 75 or less when tested per ASTM D 2843 on plastic sheets in the thickness intended for use.

    - Relative- Burning Characteristics:  As follows, when tested per ASTM D 635:

      - Class CC2 Burning rate of 2.5 inches (64 mm) per minute or less when tested on plastic glazing indicated above with a nominal thickness of 0.060 inch (1.5 mm) or the thickness intended for use.

  - Provide flat cellular polycarbonate panel fabricated from an approved cellular polycarbonate glazing (light transmitting) material with a CC1 fire rating classification per ASTM D-635 and filled with Lumira™ aerogel insulation.

    - Self-Ignition Temperature: 1110º F (599 deg. C) or greater when tested per ASTM 1929 on multi-wall cellular panel filled with Lumira™ aerogel insulation in the thickness (10mm) intended for use.

  - WARRANTY

- General:  Warranties specified in this section shall not deprive the Owner of other rights the Owner may have under other provisions of the Contract Documents and shall be in addition to and run concurrent with other warranties made by the Contractor under requirements of the Contract Documents.

- Skylight Warranty:  Provide written warranty signed by manufacturer, agreeing to repair or replace work that exhibits defects in materials or workmanship and guaranteeing weather-tight and leak-free performance.  “Defects” is defined as uncontrolled leakage of water and abnormal aging or deterioration.

  - Warranty Period:  5 years from date of Substantial Completion.

- Plastic Warranty:  Provide written warranty signed by manufacturer agreeing to repair or replace work that has or develops defects in the plastic.  “Defects” is defined as abnormal aging or deterioration.

  - Warranty Period:  5 years from date of Substantial Completion against yellowing or breakage.

- Finish Warranty:  Provide written warranty signed by manufacturer agreeing to repair or replace work with finish defects.  “Defects” is defined as peeling, chipping, chalking, fading, abnormal aging or deterioration, and failure to perform as required.

- Warranty Period for Anodized Finish:  5 year from date of shipment from the manufacturer.

- Warranty Period for 2604 Powder Finish:  10 years from date of shipment from the manufacturer.

- Warranty Period for 2605 Liquid Finish:  10 years from date of shipment from the manufacturer.    (20 years available if specified).

- Warranty Period for 2605 Powder Finish: 20 years from the date of shipment from the manufacturer.

PART 2- PRODUCTS

  - MANUFACTURERS

- Manufacturers:  Subject to compliance with requirements, provide products by Wasco Skylights part of the Velux Group, Wells, ME (800-388-0293).

- Substitutions:  Manufactures shall not be considered without prior approval in writing no later than ten (10) calendar days prior to bid.  Substitute manufacturers must have been in the custom skylight business for not less than a period of 15 years and must submit to the Architect the following:

  - List of similar projects successfully completed within the last five years.

  - Proof of financial capability.

  - Complete details of proposed skylight.

  - Complete specifications for Architect’s review.

  - MATERIALS

- Curb Frame:  High performance PVC with minimum effective thickness of 0.060 inch (1.5mm).  Provide integral condensation gutter system with corners fully welded for waterproof quality.

- Retainer Frame:  Extruded aluminum alloy 6063-T5 (min).  ASTM B 221 (ASTM B 221 M) with minimum effective thickness of 0.60 inch (1.5 mm).

- Integral Curbs:  Fabricate from double skin of 1100-H14 sheet aluminum, insulated with 1-1/2”, R5.8, EPS insulation.  Provide thermal break at top and bottom.

  - Provide .025-inch minimum thickness outer skin, mill finish. Outer skin to be .032 inch when length exceed nominal 48”

  - Provide .032-inch minimum thickness inner skin, prefinished white.

  - Curb assembly to have a minimum U-value of 0.17.

- Dome Glazing: Thermoformed acrylic glazing.

- Interior Laylite:  [10mm], [16mm] multi-wall flat cellular polycarbonate panel filled with Lumira™ aerogel insulating material.

- Thermal Break:  Fabricate skylight units with thermal chambered PVC.

- Gaskets: Structural glazing tape to form adhesive bond between PVC curb and inner laylite, between inner laylite and inner dome, and between inner dome and outer dome. Butyl tape between outer dome and extruded aluminum retainer. Gaskets form an air and water impenetrable barrier between adjacent surfaces.

- Fasteners:  Same metal as metals being fastened, or nonmagnetic stainless steel or other non-corrosive metal as recommended by manufacturer.

  - PLASTIC SKYLIGHT UNITS

- General:  Factory-assembled, deck-mounted unit consisting of impact modified plastic glazing, and polycarbonate panel filled with Lumira™ aerogel. Gasketing, and inner frame that is interlocked into the aluminum curb with self-contained flashing flange.

  - Products:  Provide deck mount model [CLD2] or [CMD2] meeting the requirements of this section.

- Curb: Manufacturer’s standard formed aluminum, including flashing flange to receive roof flashing and counter flashing.

  - Height: 12 inch, 16 inch.

- Condensation Control:  Fabricate skylight units with integral internal gutters and weeps to collect and dispose of condensation.

- Thermal Break:  Fabricate skylight units with thermal chambered PVC frame.

- Shape and Size:  As indicated by model number.

- Outer Glazing: [Dome] or [Pyramid] thermoformed:

  - [Clear], [Bronze]

- Inner Laylite:  [10mm], [16mm] multi-wall flat cellular polycarbonate panel filled with Lumira™ aerogel insulation material.

  - FABRICATION

- Framing Components:  As follows:

1.    Factory fit and assemble components.

  - Fabricate components to drain condensation and moisture occurring or migrating within skylight system to the exterior.

  - Fabricate components to accommodate expansion, contraction, and field adjustment, and to provide for minimum clearance and shimming at skylight perimeter.

  - Fabricate components to ensure that glazing is thermally and physically isolated from framing members.

  - Form shapes with sharp profiles, straight and free of defects or deformations, before finishing.

  - ALUMINUM FINISHES FOR GLAZING RETAINER (EDIT AS REQUIRED)

- General:  Comply with NAAMM “Metal Finishes Manual” recommendations for application and designations of finishes.

- Finish designations prefixed by AA conform to the system for designations of aluminum finishes established by the Aluminum Association.

  - Mill Finish:  Manufacturer’s standard mill finish.

  - 2605 Powder: Powder Coat High-Performance Architectural Coating complying with AAMA 2605. Color:______________

  - 2604 Powder: Powder Coat High-Performance Architectural Coating: complying with AAMA 2604. Color:______________

  - 2605 Liquid: Kynar Fluoropolymer Two-Coat System  (70% PVDF) complying with AAMA 2605.  Color:______________ (3coat and 4 coat available)

  - Anodized: [Clear-Anodized Finish, Class I:  AA-C22A41 complying with AAMA 611.] [Color –Anodized Finish Class I:  AA-C22A42/A44 complying with AAMA 611.  Color:______________]

PART 3- EXECUTION

  - EXAMINATION

- Examine substrates and conditions, with installer present, for compliance with requirements for installation tolerances and other conditions affecting skylight performance.

  - Proceed with installation only after unsatisfactory conditions have been corrected.

  - PREPARATION

- Metal Protection:  As follows:

  - Where aluminum will contact dissimilar metals, protect against galvanic action by paining contact surfaces with primer or by applying sealant or tape recommended by manufacturer for this purpose.

  - Where aluminum will contact concrete or masonry, protect against corrosion by painting contact surfaces with bituminous paint.

  - Where aluminum will contact pressure-treated wood, separate dissimilar materials by methods recommended by manufacturer.

  - INSTALLATION

- General:  Comply with manufacturer’s written instructions for protecting, handling, and installing skylight components.

- Coordinate with installation of roof deck and other substrates to receive skylight units.

- Coordinate with installation of vapor barriers, roof insulation, roofing, and flashing as required to assure that each element of the work performs properly and that combined elements are waterproof and weather tight.  Anchor units securely to supporting structural substrates, adequate to withstand lateral and thermal stresses as well as inward and outward loading pressures.

- Counter Flashing:  Where counter flashing is required as a component of the skylight, install to provide an adequate waterproof overlap with roofing or roof flashing (as counter flashing).  Seal with thick bead of mastic sealant, except where overlap is indicated to be left open for ventilation.

  - CLEANING AND PROTECTION

- Clean exposed metal and plastic surfaces according to manufacturer’s instructions.  Touch up damaged metal coatings.

- Clean plastic skylight units, inside and out, not more than 5 days prior to date of substantial completion.

END OF SECTION