SECTION 08620

PLASTIC GLAZED THERMALIZED IN-TANDEM (CTMD) SKYLIGHT

PART 1-GENERAL

1.1 SUMMARY

- This section includes the following:

  - CTMD model standard double dome or double pyramid domes.

[Formerly Wasco Model TSST]

- Related Sections: The following sections contain requirements that relate to this section:

  - Division Six Section “Rough Carpentry” for wood curbs and nailers.

  - Division Seven Section “Roof Accessories” for curbs, roof hatches, and smoke vents.

  - Division Seven Section “Flashing and Sheet Metal” for metal flashing for skylights.

  - Division Eight Section “Metal-Framed Skylights” for large site constructed skylights.

- Refer to roofing system sections for roofing accessories to be built into the roofing system to accommodate work of this section.

1.2 PERFORMANCE REQUIREMENTS

- General: Provide skylights capable of withstanding loads as defined by the local codes having jurisdiction where the skylights are to be installed without failure.

- Structural Loads: Plastic glazed skylights must be adequate in thickness and configuration to meet 30 PSF positive load and a 20 PSF negative load.

1.3 SUBMITTALS

- Product Data Sheet: For each type of skylight specified, include details of construction and installation, relative to applicable curb configuration.

- Samples for Selection: Manufacturer’s color charts showing a full range of colors available for each type of skylight glazing and Aluminum Finish.

1.4 QUALITY ASSURANCE

- Fire-Test-Response Characteristics: Provide glazing fabricated from sheets identical to those tested for the following fire-test-response characteristics, per ASTM test method indicated below, by UL or other testing and inspecting agencies acceptable to authorities having jurisdiction. Identify plastic sheets with appropriate markings of applicable testing and inspecting organization.

  - Self-Ignition Temperature: 651 deg F (343 deg C) or greater when tested per ASTM D 1929 on plastic sheets in the thickness intended for use.

  - Smoke density of 75 or less when tested per ASTM D 2843 on plastic sheets in the thickness intended for use.

  - Relative- Burning Characteristics: As follows, when tested per ASTM D 635:

    - Acrylic: Burning rate of 2.5 inches (64 mm) per minute or less when tested on plastic glazing indicated below with a nominal thickness of 0.060 inch (1.5 mm) or the thickness intended for use.

1.5 WARRANTY

- General:  Warranties specified in this section shall not deprive the Owner of other rights the Owner may have under other provisions of the Contract Documents and shall be in addition to and run concurrent with other warranties made by the Contractor under requirements of the Contract Documents.

- Skylight Warranty:  Provide written warranty signed by manufacturer, agreeing to repair or replace work that exhibits defects in materials or workmanship and guaranteeing weather-tight and leak-free performance.  “Defects” is defined as uncontrolled leakage of water and abnormal aging or deterioration.

  - Warranty Period:  2 years from date of Substantial Completion.

- Plastic Warranty:  Provide written warranty signed by manufacturer agreeing to repair or replace work that has or develops defects in the plastic.  “Defects” is defined as abnormal aging or deterioration.

  - Warranty Period for Acrylic:  2 years from date of Substantial Completion against yellowing.

- Finish Warranty:  Provide written warranty signed by manufacturer agreeing to repair or replace work with finish defects.  “Defects” is defined as peeling, chipping, chalking, fading, abnormal aging or deterioration, and failure to perform as required.

- Warranty Period for Anodized Finish:  5 year from date of shipment from the manufacturer.

- Warranty Period for 2604 Powder Finish:  10 years from date of shipment from the manufacturer.

- Warranty Period for 2605 Liquid Finish:  10 years from date of shipment from the manufacturer.    (20 years available if specified).

- Warranty Period for 2605 Powder Finish: 20 years from the date of shipment from the manufacturer.

PART 2- PRODUCTS

2.1 MANUFACTURERS

- Manufacturers: Subject to compliance with requirements, provide products by Wasco Skylights part of the VELUX Group, Wells, ME (800-388-0293).

- Substitutions: Manufacturers shall not be considered without prior approval in writing no later than ten (10) calendar days prior to bid. Substitute manufacturers must have been in the custom skylight business for not less than a period of 15 years and must submit to the Architect the following:

  - List of similar projects successfully completed within the last five years.

  - Proof of financial capability.

  - Complete details of proposed skylight.

  - Complete specifications for Architect’s review.

2.2 MATERIALS

- Curb Frame: Extruded aluminum alloy 6063-T5 (min.) ASTM B 221 (ASTM B 221 M) with minimum effective thickness of .094 inches. Provide integral condensation gutter system with corners fully welded for waterproof quality.

- Retainer Frame: Extruded aluminum alloy 6063-T5 (min). ASTM B 221 (ASTM B 221 M) with minimum effective thickness of 0.60 inches.

- Thermal Break:  Glass fiber reinforced polymer thermal break

- Support Structure: Minimum 1 1/2" wide at perimeter and under all rafters and purlins. Purlins can be self-supporting consult manufacturer for limitations.

  - Module size: Maximum of 8'-0" X 8'-0" grid for acrylic glazed units.

- Plastic Sheets: Monolithic, thermoformable sheets with good weather and impact resistant.

  - Acrylic: Thermoformable, and cold formable acrylic (methacrylate), Category C-2 or CC-2 Type UVA (formulated with ultraviolet absorber), with Finish 1 (smooth or polished), unless otherwise indicated.

- Fasteners: Same metal as metals being fastened, or non-magnetic stainless steel or other non-corrosive metal as recommended by manufacturer.

- Bituminous Coating: SSPC-Paint 12, solvent-type, bituminous mastic, nominally free of sulfur and containing no asbestos fibers, compounded for 15-mil (0.4 mm) dry film thickness per coating.

2.3 PLASTIC SKYLIGHT SYSTEM

- General: Curb-mounted skylight system consisting of plastic glazing, gasketing, aluminum frame with glass fiber thermal break, designed to mount on separate curb with self-contained flashing.

  - Products: Provide Model CTMS [double dome], [double pyramid dome] (EDIT AS REQUIRED) meeting the requirements of this section.

- Condensation Control: Fabricate skylight units with integral internal gutters and weeps to collect and dispose of condensation to the outside.

- Shape and Size: As indicated by model number.

- Glazing: (EDIT AS REQUIRED)

  - Outer Glazing: [Dome] or [Pyramid] thermoformed:

    - Acrylic [Clear], [# 2412 Bronze], [White].

  - Inner Glazing:  Dome thermoformed:

    - Acrylic [Clear], [# 2447 white].

  - Optional Inner Laylite: Multi-wall flat cellular polycarbonate panel filled with Lumira™ aerogel insulation material.

2.4 FABRICATION

- Framing Components: As follows:

  - Factory fit and assemble components.

  - Fabricate components to drain condensation and moisture occurring or migrating within skylight system to the exterior.

  - Fabricate components to accommodate expansion, contraction, and field adjustment, and to provide for minimum clearance and shimming at skylight perimeter.

  - Fit and secure joints by heliarc welding.

2.5 ALUMINUM FINISHES (EDIT AS REQUIRED)

- General:  Comply with NAAMM “Metal Finishes Manual” recommendations for application and designations of finishes.

- Finish designations prefixed by AA conform to the system for designations of aluminum finishes established by the Aluminum Association.

  - Mill Finish:  Manufacturer’s standard mill finish.

  - 2605 Powder: Powder Coat High-Performance Architectural Coating complying with AAMA 2605. Color:______________

  - 2604 Powder: Powder Coat High-Performance Architectural Coating: complying with AAMA 2604. Color:______________

  - 2605 Liquid: Kynar Fluoropolymer Two-Coat System  (70% PVDF) complying with AAMA 2605.  Color:______________ (3coat and 4 coat available)

  - Anodized: [Clear-Anodized Finish, Class I:  AA-C22A41 complying with AAMA 611.] [Color –Anodized Finish Class I:  AA-C22A42/A44 complying with AAMA 611.  Color:______________]

PART 3- EXECUTION

3.1 EXAMINATION

- Examine substrates and conditions with installer present, for compliance with requirements for installation tolerances and other conditions affecting skylight performance.

  - Proceed with installation only after unsatisfactory conditions have been corrected.

3.2 PREPARATION

- Metal Protection: As follows:

  - Where aluminum will contact dissimilar metals, protect against galvanic action by painting contact surfaces with primer or by applying sealant or tape recommended by manufacturer for this purpose.

  - Where aluminum will contact concrete or masonry, protect against corrosion by painting contact surfaces with bituminous paint.

  - Where aluminum will contact pressure-treated wood, separate dissimilar materials by methods recommended by manufacturer.

3.3 INSTALLATION

- General: Comply with manufacturer’s written instructions for protecting, handling, and installing skylight components.

- Coordinate with installation of roof deck and other substrates to receive skylight units.

- Coordinate with installation of vapor barriers, roof insulation, roofing, and flashing as required to assure that each element of the work performs properly and that combined elements are waterproof and weather tight. Anchor units securely to supporting structural substrates, adequate to withstand lateral and thermal stresses as well as inward and outward loading pressures.

- Counter Flashing: Where counter flashing is required as component of the skylight, install to provide an adequate waterproof overlap with roofing or roof flashing (as counter flashing). Seal with thick bead of mastic sealant, except where overlap is indicated to be left open for ventilation.

3.4 CLEANING AND PROTECTION

- Clean exposed metal and plastic surfaces according to manufacturer’s instructions. Touch up damaged metal coatings.

- Final cleaning by others.

END OF SECTION 08620